# FHDQ135S

All in one LED display

User manual

2024.10

Document version V1.5

TABLE OF CONTENTS

Dear Customer	3

Use Precautions of AIO Products	3

SAFETY	4

Safety Instructions	4

Copyright	5

Disclaimer	5

FCC Statement	5

FCC Warning	5

Declaration of Conformity for EU Countries	6

WEEE	6

Eye Health	6

INTRODUCTION	7

Package Contents	7

Product Overview	10

Remote Control	11

Interface Specifications	12

Electrical	13

USING THE DEVICE	14

Setup Wizard	14

Home Screen Overview	16

BASIC OPERATIONS	17

OSD Menu	17

Android Settings Menu	33

Additional Information	48

Supported Timing List	48

RS232 Protocol Function List	49

LED Indicator	54

Optoma Global Offices	55

# Dear Customer

Please read this manual carefully before making any adjustments and retain it for future reference.

Pictures in the manual are for reference only. If any details in this document are unclear, please contact your local representative

## Use Precautions of AIO Products

The interior design of AIO LED display screen is very sophisticated, and has no dustproof/waterproof capability. Please follow the precautions during installation and use.

- Screen storage/working environment requirements

  - Storage temperature：-20~40℃；Relative humidity：10%~65%;

  - Working temperature：0~40℃；Relative humidity：10%~80%.

- Installation requirements

  - The installation area of LED display screen is strictly prohibited from leaking or under the air conditioner outlet to avoid damage of LED devices and oxidation of circuit board caused by moisture or condensation. The installation environment humidity must be less than 60% (non-condensation state). It is recommended to install a dehumidifier in the environment to control the humidity.

  - Do not place this screen in these environments as the screen could be susceptible to the following: Metal Dust (which could cause damage to the screen and circuit boards), excessive dust and or dusty conditions (which could affect the screen performance).

- Cleaning requirements

  - During the use of the screen, if there is any accumulation of dust on the surface, it is necessary to use anti-static soft bristle brush or dust-free cloth to clean the screen. Please be careful in the cleaning process to avoid bumping and damaging the LED. Do not use a damp cloth to scrub the screen.

# SAFETY

Please follow all warnings, precautions and maintenance as recommended in this user manual.

## Safety Instructions

- Always ensure that the unit is properly earthed and power connections are secured correctly.

- Only use attachments/accessories specified by the manufacturer.

- Do not block any ventilation openings. To ensure reliable operation of the device and to protect from overheating, it is recommended to install the device in a location that does not block ventilation. Do not put the device in an enclosure such as a book case or a cabinet that restricts air flow.

- Do not use the device near water or moisture. To reduce the risk of fire and/or electric shock, do not

expose the device to rain or moisture.

- Do not install near heat sources such as radiators, heaters, stoves or any other apparatus, such as

amplifiers, that emits heat.

- Do not let objects or liquids enter the device. They may touch dangerous voltage ports or short out parts

that could result in fire or electric shock.

- Do not place the device on an unstable surface. The device may fall over, resulting in injury or damage to the device.

- Do not setup the device in places where it might be subject to vibration or shock.

- Do not use any hard or sharp object to operate the screen.

- Do not use the device if it has been physically damaged or abused. Physical damage/abuse would be (but not limited to):

  - The device has been dropped.

  - Power supply cord or plug has been damaged.

  - Liquid has been spilled on to the device.

  - The device has been exposed to rain or moisture.

  - Something has fallen in the device or something is loose inside.

- Do not attempt to service the device yourself. Opening or removing covers may expose you to dangerous voltages or other hazards. Please call Optoma before you send the device for repair.

- Disconnect the power plug from AC outlet if the device is not being used for a long period of time.

- Remove the batteries from the remote control before storage. If the batteries are left in the remote for long periods, they may leak.

- Turn off and unplug the power plug from the AC outlet before cleaning the device.

- Use a soft dry cloth with mild detergent to clean the display housing. Do not use abrasive cleaners, waxes or solvents to clean the device.

- See the device enclosure for safety related markings.

- The device should only be repaired by appropriate service personnel.

## Copyright

This publication, including all photographs, illustrations and software, is protected under international copyright laws, with all rights reserved. Neither this manual, nor any of the material contained herein, may be reproduced without written consent of the author.

© Copyright 2022

## Disclaimer

The information in this document is subject to change without notice. The manufacturer makes no representations or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. The manufacturer reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of the manufacturer to notify any person of such revision or changes.

## FCC Statement

This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation.

This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

## FCC Warning

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

- this device may not cause harmful interference, and

- this device must accept any interference received, including interference that may cause undesired operation.

###### Notice: Canadian users

This Class A digital apparatus complies with Canadian CAN ICES-3 (B)/NMB-003 (B). Remarque à l'intention des utilisateurs canadiens

Cet appareil numerique de la classe B est conforme a la norme NMB-003 du Canada.

## Declaration of Conformity for EU Countries

  - EMC Directive 2014/30/EU (Including amendments)

  - Low Voltage Directive 2014/35/EU

## WEEE

###### Disposal instructions

Do not throw this electronic device into the trash when discarding. To minimize pollution and ensure utmost protection of the global environment, please recycle it.

## Eye Health

### Calculating Breaks

It is recommended to take a 10-minute break every hour of continuous screen use.

Note: Remember to blink your eyes often to keep them healthy and avoid dryness.

### Looking at Distant Objects

During the break look at distant objects or close your eyes. This helps to relax your ciliary (focusing) muscle and avoids eye strain.

### How to Exercise Your Eyes?

- Hold your pointer finger a few inches away from your eye.

- Focus on your finger.

- Slowly move your finger away from your face, holding your focus.

- Look away for a moment, into the distance.

- Focus on your outstretched finger and slowly bring it back toward your eye.

# INTRODUCTION

## Package Contents

### Standard Accessories

| Item | Description | Photo | Qty. |
| --- | --- | --- | --- |
| 1 | Tile Removing Tool |  | 1 |
| 2 | Wi-Fi USB Dongle EZC-5201BS |  | 1 |
| 3 | Infrared Remote Control *Battery model: CR2025 |  | 1 |
| 4 | Left--Wall mount bracket |  | 2 |
| 5 | Right--Wall mount bracket |  | 2 |
| 6 | Left--Frame |  | 1 |
| 7 | Right--Frame |  | 1 |
| 8 | Top Left--Frame |  | 1 |
| 9 | Top Right Frame |  | 1 |
| 10 | Logo Fixer |  | 1 |
| 11 | Cabinet Fixer |  | 8 |
|  | M3*6 Screw with washer |  | 26 |
|  | M5*10 Screw |  | 7 |
|  | M3*6 Screw |  | 34 |
|  | M4*8 Screw |  | 9 |
|  | M4*10 Screw |  | 18 |
|  | Washer |  | 10 |
|  | M5*25 Screw |  | 18 |
|  | M3*20 Screw |  | 2 |
|  | FPC cable (430mm) |  | 3 |
|  | AC Power Cord (EU) |  | 1 |
|  | AC Power Cord (UK) |  | 1 |
|  | AC Power Cord (USA) |  | 1 |
|  | L-shaped hex key 3# |  | 1 |
|  | L-shaped hex key 4# |  | 1 |
|  | L-shaped hex key 5# |  | 1 |
|  | Screw driver 1# |  | 1 |
|  | Magnet adjustment tool |  | 2 |
|  | Assist Handle |  | 2 |

Note: For warranty information, please visit www.optoma.com.

### Spare Parts

| Item | Description | Photo | Qty. |
| --- | --- | --- | --- |
| 1 | LED tile |  | 4 |
| 2 | LEDs |  | 500 |
| 3 | LED Driver IC |  | 50 |

## Product Overview

### Front View

### Rear View

## Remote Control

| ID | Button | Description |
| --- | --- | --- |
| 1 | Power button | Power button to turn on/off the unit |
| 2 | Source | Switch and select the input source |
| 3 | Display Mode | Jump to display mode setting |
| 4 | PIP Layout | Jump to PIP layout setting |
| 5 | LED Brightness | Jump to the LED brightness setting |
| 6 | Up | Direction key Up |
| 7 | Left | Direction key Left |
| 8 | Right | Direction key Right |
| 9 | Down | Direction key Down |
| 10 | Enter | Confirm the selection in OSD |
| 11 | Menu | On Screen Display (OSD) menu |
| 12 | Mute | Jump to Volume mute setting |
| 13 | Home | Launcher home page |
| 14 | Back | Back to previous page in the Menu |
| 15 | Information | Device information Menu |

*Battery spec: CR2025 Lithium Coin battery 3V

## Interface Specifications

| Item | Port | IN/OUT | SPEC and Function |
| --- | --- | --- | --- |
| 1 | LAN | IN | Sender service |
| 2 | HDMI 2 Output | OUT | HDMI: 2.0, 4K output |
| 3 | Power Key | IN | Power On/Off |
| 4 | 3G-SDI | IN | Serial digital Image input (1080p) |
| 5 | HDMI_1 Loop | OUT | HDMI 1 loop through output, 4K |
| 6 | HDMI_1 | IN | HDMI: 2.0, 4K |
| 7 | DP | IN | DP: 1.2a, 4K |
| 8 | HDMI_2 | IN | HDMI: 2.0, 4K |
| 9 | HDMI_3 | IN | HDMI: 2.0, 4K |
| 10 | 3G-SDI | OUT | Genlock support |
| 11 | IR IN | IN | External IR input (3.5mm) TRS |
| 12 | Audio Output | OUT | HIFI stereo Line output (3.5mm) TRS, 2Vrms |
| 13 | USB (Type A) | IN/OUT | USB 3.0 Type A Host for media playback |
| 14 | USB (Type A) | IN/OUT | USB 2.0 Type A OTG for W-Fi dongle or others |
| 15 | LAN | IN/OUT | RJ45 for Telnet |
| 16 | RS232_2 | IN/OUT | Multi-RS232 CLI External Control |
| 17 | RS232_1 | IN/OUT | Multi-RS232 CLI External Control |
| 18 | USB (Type B) | (Reserved) | (Reserved) |
| 19 | USB (Type A) | (Reserved) | (Reserved) |

## Electrical

| Electrical Characteristics for FHDQ135S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| At 100V AC Input |  |  |  |  | At 220V AC Input |  |  |  |  |
| Parameter | Unit | Rating nom. | Max. | Parameter |  | Unit | Rating nom. | Max. |
| Input current | A | 6.00 | 15.00 | Input current |  | A | 3.64 | 9.10 |
| Input freq. | Hz | 50 – 60 | 65 | Input freq. |  | Hz | 50 – 60 | 65 |
| Power | VA | 600 | 1500 | Power |  | VA | 800 | 2000 |

Circuit Protection

Each section of the screen should be protected by a Circuit Breaker to protect against high fault currents, and a Residual Current Device (RCD) to detect earth leakage currents

Earth Leakage Considerations

Each LED display has earth leakage current contributions from twenty-six switched-mode power supplies and a filter. The total earth leakage current per LED display can be up to 26.35mA at 240V. If a 30mA RCD is used, then the maximum number of LED displays per section should be safely limited to 1.

Inrush-current and over-current considerations

The inrush current of the switched mode power supplies used to provide LED and logic power to each LED display is specified by the manufacturer as 1330A at 230volts for a duration of 3mS. However, maximum current in the steady-state for a LED display is considerably less: 12.50A at 240 volts and 15.00A at 200 volts. The circuit breaker used to control a section of the screen must be able to handle the short-term inrush current at switch-on, without causing unnecessary disconnection of the supply, and be able to detect excess currents due to faults within the equipment, during normal operation. 20A Circuit Breaker is recommended.

Earthing

Each LED display is separately connected to the AC power distribution system’s earth through green/yellow wires connection. All exposed metalwork is also connected to this earth.

# USING THE DEVICE

## Setup Wizard

The Setup Wizard will appear on the screen when you turn on your device for the first time.

Note: This device is equipped with remote control to operate your device.

- Select the language for the on-screen display (OSD) menu. Select Next to continue.

- If you want to connect your device to the Internet wirelessly, select one of the available networks and select Connect. Otherwise, select Skip.

###### IMPORTANT!

  - To connect to a wireless network, make sure the Wi-Fi dongle is properly plugged into the Wi-Fi dongle slot. Refer to page 12.

  - If you are connecting to a secure network, you will need to enter the password when connecting for the first time only.

- Configure the date and time related settings. Select Next to continue

- Read Optoma’s legal policies, including Terms and conditions of use, Privacy policy, Cookie policy, and Open source. Check the I agree to Terms and conditions, Privacy policy, Cookies policy and Open-source radio button and then select Next to complete the setup. The device is now ready to use.

## Home Screen Overview

Home screen is displayed every time you start the device.

Sign in to White Board	Wireless display

verification code from

Current date and time

Apps shortcuts

Display Share

Customizable icons

Apps list

Wi-Fi Hotspot status (only appears when Wi-Fi Hotspot connection is established)

OSD menu

Wi-Fi connection status (only appears when Wi-Fi connection is established)

LAN connection status (only appears when the device is connected to LAN)

Settings

menu

USB flash drive (only appears when the USB flash drive is plugged into the USB port)

## OSD Menu

To open the OSD menu, do any of the following

- Press the Menu or Input button on the remote control.

- on the Home screen.

Main menu	Submenu                                      Settings

Note:

To configure the OSD menu settings, simply select the menu item on the screen and use the screen controls to adjust its value or set the setting. The selected menu item is highlighted in Blue.

To move to the previous item in the menu, select   or press Back on the remote control.

To close the OSD menu, press Back continuously (if necessary) on the remote control.

Shortcut Menu

Shortcut  menu is the first OSD menu.

Select the input source. Those input sources that are available are marked with the DOT ● symbol.

The shortcut page is available to quickly adjust the volume level and LED brightness level.

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Volume level | Adjusts the volume. | 0 to 100 | 50 |
| LED Brightness level | Adjusts the LED Brightness. | 1 to 100 | 50 |

Note: The OPS option is only available if the PC module is properly installed into the OPS slot of the device.

Configuring the Image Settings

In the OSD menu, select  to open the Image menu, then complete the following steps:

Display Mode

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Display Mode | Optimizes for displaying images under 4 settings | Presentation Bright Cinema HDR User | Presentation |

Note:

Image settings can only be adjusted in HDMI, 3G-SDI, DP, OPS input. This will be grayed out and cannot be adjusted while in the Android Source.

Gamma and Color Temperature is the setting to select LED gamma, and LED Color Temperature, and can be adjusted in each input and display mode except for HDR mode.

If the Display Mode setting is set to User, you can customize image settings by adjusting the brightness, contrast, sharpness, color space and color settings based on your preferences.

Press slider bar to the right or to the left to adjust the Brightness/ Contrast / Sharpness level

Press the drop down menu to select the Display Mode / Gamma /Color Temperature/ Color Space option

| Brightness | Adjusts the intensity of the image. 0 is minimum brightness. 100 is maximum brightness. | 0 to 100 | 50 |
| --- | --- | --- | --- |
| Contrast | Adjusts the degree of difference between the lightest and darkest parts of the image and changes the amount of black and white in the image | 0 to 100 | 50 |
| Sharpness | Adjusts the sharpness of the image. | 0 to 8 | 0 |
| Gamma | Adjusts the gamma value to obtain a better image contrast for the input. | 1.8/2.0/2.2/2.4 | 2.4 |
| Color Temperature | Adjusts the color temperature to 4 modes. | 3200K 6500K 9300K 12000K | 6500K |
| Color Space | Sets the color space of the input source of limited range RGB/YUV. | Auto RGB (0~255) RGB (16~235) YUV (0~255) YUV (16~235) | Auto |
| Reset | Reset all image setting to default |  |  |

Color Setting

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Color setting | Use these settings for advanced adjustment of the individual color | Color Tint RGB Gain/Bias |  |

Click the bar to adjust the Color/ Tint / Color temperature

| Sub- item | Description | Options | Default |
| --- | --- | --- | --- |
| Color | Adjusts the color saturation | 0 to 100 | 50 |
| Tint | Adjusts the red-green color balance in the image. | 0 to 100 | 50 |

Note: Tint level and color level can be adjusted only if the Display Mode is set to User. In        other display modes these parameters are preset and cannot be adjusted.

Select RGB Gain/ Bias and to adjust more following settings

| RGB Gain/ Bias | Adjusts RGB Gain and Bias | Red Gain (0~100) Green Gain (0~100) Blue Gain (0~100) Red Bias (0~100) Green Bias (0~100) Blue Bias (0~100) | 50 |
| --- | --- | --- | --- |
| Reset | Reset RGB Gain/ Bias to Default Value |  |  |

Configuring the Display Settings

In the OSD menu, select   to open the Display menu, then complete the following steps:

Select Input/Aspect Ratio and select one of the available options, select LED Brightness bar to adjust the brightness level

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Input | Select an input source | Android HDMI1 HDMI2 HDMI3 DP 3G-SDI OPS | Android |
| Aspect Ratio | Displays an image with the detected size, or resizes the image by maximizing either the height, width, both, or resizes to the maximum size possible while keeping the original aspect ratio. | 16:9 4:3 Auto | Auto |
| LED Brightness | Adjusts the intensity of the LED | 1 to 100 | 50 |

Press Test Pattern and select one of the available options.

| Test Pattern | Displays or hides the test pattern | 0: Off 1: Red Curtain 2: Green Curtain 3: Blue Curtain 4: Black Curtain 5: White Curtain 6: H Ramp 7: V Ramp 8: H Color Bar 9: V Color Bar 10: Checker 11: Grid 12: Slider Bar | Off |
| --- | --- | --- | --- |

Configuring the Advanced Settings

In the OSD menu, press  to open the Advanced setting menu,

Drag the PIP mode to expand the options Custom 1, Custom 2 and Custom 3

Select Custom 01-03 to set up the following options

Change PIP Template name by selecting

| Sub- item | Description | Options | Default |
| --- | --- | --- | --- |
| Edit Name | Editing name of template and can be applied to custom 01-03. | Custom defined template name |  |

Drag the PIP 1 or PIP 2 to expand the options of PIP 1 or PIP 2

Change settings of Main input/ Input/Size of PIP 1 and PIP 2

| Sub- item | Description | Options | Default |
| --- | --- | --- | --- |
| Main Input | Main Input | HDMI1 HDMI2 HDMI3 DP 3G-SDI OPS | HDMI1 |
| Input | Select input source of PIP 1 input or PIP 2 input. | HDMI1 HDMI2 HDMI3 DP 3G-SDI OPS | HDMI 2(PIP 1) HDMI 3(PIP2) |
| Size | Adjust size of the PIP screen | Small Medium Large Custom: Width: 64-1920 Pixel Height: 64-1080 Pixel | Medium |

Note: Select the Width to scroll the value (64-1920) to adjust the image width manually.

Select the Height to scroll the value (64-1080) to adjust the image height manually

Position

| Position | Adjust location of the PIP screen | Top Left Top Right Center Bottom Left Bottom Right Custom: H Position:0-1920 Pixel V Position:0-1080 Pixel | Bottom Left (PIP1) Bottom Right (PIP2) |
| --- | --- | --- | --- |

Note: Select the H Position to scroll the value (0-1920) to adjust the image horizontal position manually

Select the V Position to scroll the value (0-1080) to adjust the image vertical position manually

PIP1 and PIP 2 conflict table

| Only PIP 1 | PIP 1 with PIP 2 Bottom Right | PIP 1 with PIP 2 Top Left |
| --- | --- | --- |
| PIP 1 with PIP 2 Top Right | PIP 1 with PIP 2 Bottom Left | H Position and V Position |

User could define PIP 1 or PIP 2 setting first, and choose the location of PIP 1 or PIP 2 shown on the device.

When main input is HDMI 1

| Main | HDMI1 | PIP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI1 | HDMI2 | DP | 3G-SDI | HDMI3 | OPS |
| PIP2 | HDMI1 |  |  |  |  |  |  |
|  | HDMI2 |  |  |  |  |  |  |
|  | DP |  |  |  |  |  |  |
|  | 3G-SDI |  |  |  |  |  |  |
|  | HDMI3 |  |  |  |  |  |  |
|  | OPS |  |  |  |  |  |  |

Note:

1.  presumed that when setting PIP 1 as HDMI1, then HDMI 1, HDMI 2, DP, HDMI 3, and OPS are available for PIP2.

2.  presumed that when setting PIP 1 as HDMI 2, then HDMI 1, HDMI 2, HDMI 3, and OPS are available for PIP2.

When main input is HDMI 2.

| Main | HDMI2 | PIP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI1 | HDMI2 | DP | 3G-SDI | HDMI3 | OPS |
| PIP2 | HDMI1 |  |  |  |  |  |  |
|  | HDMI2 |  |  |  |  |  |  |
|  | DP |  |  |  |  |  |  |
|  | 3G-SDI |  |  |  |  |  |  |
|  | HDMI3 |  |  |  |  |  |  |
|  | OPS |  |  |  |  |  |  |

When main input is HDMI 3.

| Main | HDMI3 | PIP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI1 | HDMI2 | DP | 3G-SDI | HDMI3 | OPS |
| PIP2 | HDMI1 |  |  |  |  |  |  |
|  | HDMI2 |  |  |  |  |  |  |
|  | DP |  |  |  |  |  |  |
|  | 3G-SDI |  |  |  |  |  |  |
|  | HDMI3 |  |  |  |  |  |  |
|  | OPS |  |  |  |  |  |  |

When main input is OPS

| Main | OPS | PIP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI1 | HDMI2 | DP | 3G-SDI | HDMI3 | OPS |
| PIP2 | HDMI1 |  |  |  |  |  |  |
|  | HDMI2 |  |  |  |  |  |  |
|  | DP |  |  |  |  |  |  |
|  | 3G-SDI |  |  |  |  |  |  |
|  | HDMI3 |  |  |  |  |  |  |
|  | OPS |  |  |  |  |  |  |

When main input is DP

| Main | DP | PIP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI1 | HDMI2 | DP | 3G-SDI | HDMI3 | OPS |
| PIP2 | HDMI1 |  |  |  |  |  |  |
|  | HDMI2 |  |  |  |  |  |  |
|  | DP |  |  |  |  |  |  |
|  | 3G-SDI |  |  |  |  |  |  |
|  | HDMI3 |  |  |  |  |  |  |
|  | OPS |  |  |  |  |  |  |

Note: HDMI 2 is not available in both PIP1 and PIP 2.

When main input is 3G-SDI

| Main | 3G-SDI | PIP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HDMI1 | HDMI2 | DP | 3G-SDI | HDMI3 | OPS |
| PIP2 | HDMI1 |  |  |  |  |  |  |
|  | HDMI2 |  |  |  |  |  |  |
|  | DP |  |  |  |  |  |  |
|  | 3G-SDI |  |  |  |  |  |  |
|  | HDMI3 |  |  |  |  |  |  |
|  | OPS |  |  |  |  |  |  |

PBP Mode (Picture By Picture): Dual Mode(Two Layers) / Triple Mode(Three Layers) / Quad Mode(Four Layers)

User can enable Dual mode/Triple mode or Quad mode and select the input source for each Input. It is the source of PBP layers

Note: If the input source conflict with higher layers, it will output black screen

Select Dual Mode layouts:

Select Input source for current selected layout of dual mode:

Select Triple Mode layouts:

Select Input source for current selected layout of Triple Mode:

Select Input source for current selected layout of Quad Mode:

Multiple Output

Second Display: the selected output source will output to HDMI 2 output

Click to enable Second display and select output source.

Splicing Mode – synchronization of multiple outputs

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Splicing Mode | Enable 2x1 splicing screen | Off 2x1 | Off |
| Power On all devices | Turn on all device which connect together |  |  |

Select I/O lock to synchronize outputs

| I/O Lock | Input and output synchronization. | Off Source Lock Genlock | Off |
| --- | --- | --- | --- |

Note:

1.Source Lock: output reference input

2.Genlock: the output refresh rate will follow the vertical sync of the signal connected to the Genlock BNC connector. Genlock is achieved when the Genlock vertical sync rate matches the vertical sync rate set in the output menu.

Set show Device ID to check ID of the device

Set Device ID

| Set Device ID | Setting device ID | Auto Manual | Auto |
| --- | --- | --- | --- |
| Primary Screen | Set Primary Device ID | 01-99 | 1 |
| Secondary Screen | Set Secondary Device ID | 01-99 | 2 |

Note: PIP, PBP, Second Display, Splicing Mode, and Test Pattern conflict with each other. Turning on one mode will automatically turn off other modes.

Configuring Audio Settings

In the OSD menu, press  to open the Audio menu, then complete the following steps:

Press Mute to mute or unmute audio.

Drag the Volume slider bar to adjust the volume level.

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Source | Select to audio source come from the select input. Select auto will output the audio come from main input | Auto HDMI 1 HDMI 2 HDMI 3 DP 3G-SDI OPS | Auto |
| Mute | -Mute/Unmute all audio (Speaker and 3.5 mm jack) | Off On | Off |
| Volume | -Adjust volume level | 0-10 | 5 |
| Internal Speaker | Enable speaker audio | Off On | On |

Note: if audio source conflicts with other selected video sources, will have no sound output.

Viewing Device Information

In the OSD menu, select  to open the Information menu, then complete the following steps:

Select Device to view Ethernet IP address, Wi-Fi IP address, Device ID, S/N, LED Display Hours, AC In Voltage, Power mode (Standby), Android Version, FW System, FW sender, FW LED module.

Select Display to view Input Source, Resolution, Refresh Rate, Display Mode, Color Depth, Color Format.

## Android Settings Menu

To open the Settings menu, select  on the Home screen.

Note: Settings can be modified only in Android mode.

Main menu	Submenu	Settings

###### Note:

  - To configure Android settings, simply press the menu item on the screen and use the screen controls to adjust its value or set the setting. The selected menu item is highlighted in Blue.

  - To move to the previous item in the menu, press   or press Back on the remote control.

  - To close the OSD menu press Back continuously (if necessary) on the remote control.

### Personalizing the System

In the Settings menu, select  to open the Personalized menu, then complete the following steps:

- Press Wallpaper and select a pre-loaded image as a wallpaper or an image from your gallery

If you select a pre-loaded wallpaper from the Pictures folder, the setting will take effect immediately.

If you prefer to use a customized wallpaper, follow the steps:

- In the Custom folder press .

- Select your preferred image from the local or external storage and tap Import. You can import up to 10 images.

- In the Custom folder tap on the image you wish to use as a wallpaper. The setting takes effect immediately.

Note: To delete images in the Custom folder, select . Then select the image and press Delete.

- Type: Select the image type.

- Change pictures: Select the slideshow interval from 5mins to 1 day. By default, the slideshow is off.

- Select Input Name to rename the input. Press the input name you wish to change, then type a new name and

confirm name modification by selecting OK at the prompt.

- Select Menu Settings to set the location and set close timer of menu.

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Menu location | Set location for the menu shown on screen | - Top Left - Top Right - Center - Bottom Left - Bottom Right | Center |
| Menu timer | Set duration for menu shown on the screen | Off to 60sec | 10 sec |

### Configuring Network Settings

In the Settings menu, select  to open the Network menu, then complete the following steps:

- Select Ethernet to configure LAN settings and view the current connection status. In the IP settings section select the network type between Static and DHCP. If you set the network type as Static then press  and enter the connection settings, such as IP address, gateway, network prefix length, and more.

IMPORTANT! To connect to a wired network, make sure the device is connected to LAN. For more information, refer to page 12.

- Select Wi-Fi to connect the device to the Wi-Fi network and view current connection status. Make sure the

Wi-Fi function is enabled. In the Available network list, select your preferred Wi-Fi network, for example XXXX. Provide an access password if needed.

IMPORTANT! To connect to a Wi-Fi network, make sure the Wi-Fi dongle is properly installed in the Wi-Fi dongle slot. For more information, refer to page 12.

Note: If your preferred Wi-Fi network is not visible in the list, press  to refresh the list. To enter the Wi-Fi network manually, press .

- Enable Portable hotspot feature to share the Wi-Fi network to other wireless devices. Modify the network name and set an access password if necessary.

Note: For a secure connection, set the Wi-Fi encryption (Security field) as WPA2-PSK. Then enter the access password and press OK.

- Select Hostname to set up host name

### Configuring Device Settings

In the Settings menu, select  to open the Device menu, then complete the following steps:

- Select Device ID to assign a new ID to the device. To confirm, select OK.

- Select Storage to view storage usage

- Select Apps to view apps information, clear apps data and cache, and force stop apps.

- Select an app in the list to force stop it (select Force stop), view its storage and clear its data (select Clear data) and Cache (press Clear cache), view app version and set permissions.

### Configuring Security Settings

In the Settings menu, press  to open the Security menu, then complete the following steps:

- Select Screen Lock Password to set screen lock password. Enter a 4-digit password twice at the prompt and then press OK to confirm.

Note: To remove the screen lock password, press Clear Screen Lock Password (visible only if the password is set) and enter the current password. Then press OK to confirm.

### Configuring System Settings

In the Settings menu, press  to open the System menu, then complete the following steps:

- Select Date and Time to configure date and time related settings. Choice any of the following:

- Auto Date and Time to automatically set the time from your UTC.

- Time Zone to select/change the time zone for your area.

- Use 24-hour format (13:00) to enable or disable displaying time in 24-hour format.

- Press Language to select one of the available language options: English, French, Spanish, Traditional Chinese, Simplified Chinese, Portuguese, German, Dutch, Polish, Russian, Czech, Danish, Swedish, Italian, Turkish, Arabic, Romanian, Hungarian, Finnish, or Norwegian.

- Select Keyboard to view and manage keyboards.

| Menu item | Description | Options | Default |
| --- | --- | --- | --- |
| Current Keyboard |  | - Lean back keyboard - keyboard (AOSP) | Lean back keyboard |
| Virtual Keyboard |  | - Android keyboard (AOSP) - Lean back keyboard Key board setting (use system language to choose your input method) |  |
| Physical Keyboard |  |  | Default |

- Select Startup and Shutdown to configure settings related to starting and shutting down the device. Select any of the following:

- Startup Input Source item to select one of the available startup input options: (Last Shutdown Input/Android/HDMI1/HDMI2/HDMI3/DP/3G-SDI/OPS).

- Timers   to set a startup and shutdown timer. To create a schedule, do the following:

  - To enable startup timer and/or shutdown timer, select the option button next to Startup timer and

Shutdown timer.

  - Set the time and select the day(s) when you want the device to follow this startup or shutdown schedule.

  - Press Add to save the timer.

  - Enable (or disable) the new schedule in Timers menu.

###### Note:

    - Press  to modify or remove the schedule.

    - You can set up to 3 schedules.

- Select Power to configure power settings. Press any of the following:

- Signal Power On to enable or disable starting up the device when an input signal is detected.

- Direct Power On to enable or disable starting up the device from the power source.

Note: Direct power on feature aids installations that rely on power-on timers.

- Auto Power Off Select one of the available time intervals to automatically power off the device when there’s no input source found. Off (function disabled), 1, 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55 or 60 (minutes).

- Select Power Mode Standby item to select one of the available options: Eco or Active.

###### Note:

  - OMS and LAN control remote power on functions will only work when Power Mode (Standby) is set to

Active.

  - Power Mode Standby Active, consumes higher power as the system only turns off the display. The mainboard is still active and is listening for power on commands from LAN or OMS. If a remote power on requirement is not needed, we suggest that the Power Mode Standby is set to ECO.

  - It is recommended that during long periods of inactivity that the device is set to Power Mode Standby ECO and to turn off the LED for a period of time. This will help the system to release built up system resources.

- Select Update to configure upgrading options. Press any of the following

- OTA (Over-The-Air) Software Upgrade to enable or disable upgrading software over Internet. Press Auto to enable the system to check for firmware upgrades automatically every time the device is powered on and connected to the Internet. If the Auto option is disabled, the system will not check for firmware

upgrades automatically.

- To update the system manually, do the following:

- Select UPDATE at the prompt.

- Select Download at the System Update window.

- Select UPDATE to start the update process.

Wait until the update process is complete.

Note: Upgrading software over the Internet requires network connection.

- USB Software Upgrade to update the system from a USB flash disk.]

Note: To proceed with upgrading software, save the upgrade file to the USB flash disk first, then connect the disk to a USB 2.0 port on the Device. Open Advanced Settings  System  Update  USB Software Upgrade and tap Start at the prompt.

Note: When upgrading the device’s firmware from a USB storage, follow these steps:

Ensure that the USB flash disk is formatted to FAT32.

Firstly, extract the upgrade file.

Secondly, copy the extracted firmware file to the root directory of the USB flash disk.

###### IMPORTANT!

  - During the update process, either OTA (over the air) or from a USB storage, DO NOT power off the Device and DO NOT remove the USB dongle from the Device. Doing so will damage the system.

- Select Legal to view Terms and Conditions of Use, Privacy policy, Cookie policy, and Open Source Library

### Optoma Management Suite (OMS)™

OMS is a large display management software that operates through LAN or over Internet. It simplifies the

management of the device remotely.

OMS has the following features:

- Accessible from anywhere with Internet access at any time.

- Device monitoring, alerts, control, scheduling, broadcasting, and more.

- Over-the-air firmware updates.

To start using OMS over LAN, complete the following steps:

- Connect the device and your PC to the same LAN.

Open the browser and download the OMS app to your PC from the Optoma web page:

Optoma Management Suite - OMS - Advanced remote management solution for audio visual displays (optomaeurope.com)

- Install OMS to your PC.

- Double-click on .

The OMS control page will open.

- Click on Add  Device.

- Select Automatic Search and click Search to search for devices automatically.

Note: If the device cannot be found automatically, then click Manual Search to search the device by the IP address or click Range Search to search the devices within the range of IP addresses.

- Select the device and click Add.

- Select the device and then use the screen controls to display a message on the screen of the selected device, power on/off the device, enable/disable wake on LAN, select an input source, enable/disable AV mute mode, mute or unmute the device, monitor the device’s status, usage time, mute status, temperature, and more.

Note: For enhanced device management, you can group similar devices. To create a device group, click

Add  Group. Then add the selected devices to the group.

# Additional Information

## Supported Timing List

| Format | Resolution | Frame Rate(Hz) | SDI | HMDI 1 (2.0) | HMDI 2 (2.0) | HMDI 3 (2.0) | OPS (2.0) | DP (1.2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SVGA | VESA 800 x 600 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| XGA | VESA 1024 x 768 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| 720p | 1280 x 720 | 30 | V | V | V | V | V | V |
|  |  | 50 | V | V | V | V | V | V |
|  |  | 59.94 | V | V | V | V | V | V |
|  |  | 60 | V | V | V | V | V | V |
| WXGA | VESA 1280 x 800 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| SXGA | VESA 1280 x 1024 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| FWXGA | VESA 1366 x 768 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| SXGA+ | VESA 1400 x 1050 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| WXGA+ | VESA 1440 x 900 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| UXGA | VESA 1600 x 1200 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| 1080i | 1920 x 1080 | 50 | V | V | V | V | V | V |
|  |  | 59.94 | V | V | V | V | V | V |
|  |  | 60 | V | V | V | V | V | V |
| 1080p |  | 25 | V | V | V | V | V | V |
|  |  | 30 | V | V | V | V | V | V |
|  |  | 50 | V | V | V | V | V | V |
|  |  | 59.94 | V | V | V | V | V | V |
|  |  | 60 | V | V | V | V | V | V |
| UWFHD | 2560x1080 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| WQHD | 2560x1440 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| WQXGA | 2560x1600 | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |
| 4K UHD | 3840x2160 | 30 | X | V | V | V | V | V |
|  |  | 50 | X | V | V | V | V | V |
|  |  | 59.94 | X | V | V | V | V | V |
|  |  | 60 | X | V | V | V | V | V |

## RS232 Protocol Function List

Note: DEVICE supports RS232 and RJ45 control command.

### RS232 Pin Assignments

9	8	7	6

5	4	3	2	1

| PIN No. | Spec (from display side) |
| --- | --- |
| 1 | N/A |
| 2 | TXD |
| 3 | RXD |
| 4 | N/A |
| 5 | GND |
| 6 | N/A |
| 7 | N/A |
| 8 | N/A |

### RS232 RJ45 Control Connection

| RS232 Settings |  |  | LAN Control Settings |  |
| --- | --- | --- | --- | --- |
| Baud Rate | 9600 |  | IP | Machine IP |
| Data Bits | 8 |  | Port | 23 |
| Parity | None |  |  |  |
| Stop Bits | 1 |  |  |  |
| Flow Control | None |  |  |  |

~

X

X

X

X

X

X

X

CR

Lead Code

Device ID

(where 01 indicates Device ID = 01)

Command ID

Space	Variable	Carriage return

### SET Command

| Type | Category | Parameter | CMD |  | Hex Code CMD | Response |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Success | Failed |
| SET | Power | Power off | ~xx00 n | n=0 | 7E 30 30 30 30 20 30 0D | P | F |
|  |  | Power on | ~xx00 n | n=1 | 7E 30 30 30 30 20 31 0D | P | F |
|  |  | Restart | ~xx00 n | n=3 | 7E 30 30 30 30 20 33 0D | P | F |
|  | Power Mode (standby) | Eco. | ~xx114 n | n=0 | 7E 30 30 31 31 34 20 30 0D | P | F |
|  |  | Active |  | n=1 | 7E 30 30 31 31 34 20 31 0D | P | F |
|  | Contrast | 0 ~ 100 | ~xx22 n | n=0-100 | 7E 30 30 32 32 20 30 0D to 7E 30 30 32 32 20 31 30 30 0D | P | F |
|  | Brightness | 0 ~ 100 | ~xx21 n | n=0-100 | 7E 30 30 32 31 20 30 0D to 7E 30 30 32 31 20 31 30 30 0D | P | F |
|  | Gamma | 1.8 | ~xx35 n | n=5 | 7E 30 30 33 35 20 35 0D | P | F |
|  |  | 2.0 |  | n=6 | 7E 30 30 33 35 20 36 0D | P | F |
|  |  | 2.2 |  | n=7 | 7E 30 30 33 35 20 37 0D | P | F |
|  |  | 2.4 |  | n=12 | 7E 30 30 33 35 20 31 32 0D | P | F |
|  | Image Reset |  | ~xx509 n | n=1 | 7E 30 30 35 30 39 20 31 0D | P | F |
|  | Volume | 0 ~ 10 | ~xx81 n | n=0-10 | 7E 30 30 38 31 20 30 0D to 7E 30 30 38 31 20 31 30 30 0D | P | F |
|  | Mute | Off | ~xx80 n | n=0 | 7E 30 30 38 30 20 30 0D | P | F |
|  |  | On |  | n=1 | 7E 30 30 38 30 20 31 0D | P | F |
|  | Input Source | HDMI1 | ~xx12 n | n=1 | 7E 30 30 31 32 20 31 0D | P | F |
|  |  | HDMI2 |  | n=15 | 7E 30 30 31 32 20 31 35 0D | P | F |
|  |  | HDMI3 |  | n=16 | 7E 30 30 31 32 20 31 36 0D | P | F |
|  |  | DP |  | n=20 | 7E 30 30 31 32 20 32 30 0D | P | F |
|  |  | 3G-SDI |  | n=22 | 7E 30 30 31 32 20 32 32 0D | P | F |
|  |  | OPS |  | n=25 | 7E 30 30 31 32 20 32 35 0D | P | F |
|  |  | Android |  | n=24 | 7E 30 30 31 32 20 32 34 0D | P | F |
|  | Aspect Ratio | 4:3 | ~xx60 n | n=1 | 7E 30 30 36 30 20 31 0D | P | F |
|  |  | 16:9 |  | n=2 | 7E 30 30 36 30 20 32 0D | P | F |
|  |  | Auto |  | n=7 | 7E 30 30 36 30 20 37 0D | P | F |
|  | Language | English | ~xx70 n | n=1 | 7E 30 30 37 30 20 31 0D | P | F |
|  |  | Français |  | n=3 | 7E 30 30 37 30 20 33 0D | P | F |

| Type | Category | Parameter | CMD |  | Hex Code CMD | Response |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Success | Failed |
| SET | Language | Español | ~xx70 n | n=5 | 7E 30 30 37 30 20 35 0D | P | F |
|  |  | Traditional Chinese |  | n=13 | 7E 30 30 37 30 20 31 33 0D | P | F |
|  |  | Simplified Chinese |  | n=14 | 7E 30 30 37 30 20 31 34 0D | P | F |
|  |  | Português |  | n=6 | 7E 30 30 37 30 20 36 0D | P | F |
|  |  | German |  | n=2 | 7E 30 30 37 30 20 32 0D | P | F |
|  |  | Dutch |  | n=8 | 7E 30 30 37 30 20 38 0D | P | F |
|  |  | Polish |  | n=7 | 7E 30 30 37 30 20 37 0D | P | F |
|  |  | Russia |  | n=17 | 7E 30 30 37 30 20 31 37 0D | P | F |
|  |  | Czech |  | n=19 | 7E 30 30 37 30 20 31 39 0D | P | F |
|  |  | Danish |  | n=24 | 7E 30 30 37 30 20 32 34 0D | P | F |
|  |  | Swedish |  | n=9 | 7E 30 30 37 30 20 39 0D | P | F |
|  |  | Italian |  | n=4 | 7E 30 30 37 30 20 34 0D | P | F |
|  |  | Turkish |  | n=22 | 7E 30 30 37 30 20 32 32 0D | P | F |
|  |  | Arabic |  | n=20 | 7E 30 30 37 30 20 32 30 0D | P | F |
|  |  | Romanian |  | n=27 | 7E 30 30 37 30 20 32 37 0D | P | F |
|  |  | Hungarian |  | n=18 | 7E 30 30 37 30 20 31 38 0D | P | F |
|  |  | Finnish |  | n=11 | 7E 30 30 37 30 20 31 31 0D | P | F |
|  |  | Norge |  | n=10 | 7E 30 30 37 30 20 31 30 0D | P | F |
|  | Display Mode | Presentation | ~xx20 n | n=1 | 7E 30 30 32 30 20 31 0D | P | F |
|  |  | Bright |  | n=2 | 7E 30 30 32 30 20 32 0D | P | F |
|  |  | Cinema |  | n=3 | 7E 30 30 32 30 20 33 0D | P | F |
|  |  | HDR |  | n=21 | 7E 30 30 32 30 20 32 31 0D | P | F |
|  |  | User |  | n=5 | 7E 30 30 32 30 20 35 0D | P | F |
|  | Color | 0 ~ 100 | ~xx45 n | n=0-100 | 7E 30 30 34 35 20 30 0D to 7E 30 30 34 35 20 31 30 30 0D | P | F |
|  | Color Temp | 9300K | ~xx36 n | n=2 | 7E 30 30 33 36 20 32 0D | P | F |
|  |  | 6500K |  | n=1 | 7E 30 30 33 36 20 31 0D | P | F |
|  |  | 3200K |  | n=4 | 7E 30 30 33 36 20 34 0D | P | F |
|  |  | 12000K |  | n=3 | 7E 30 30 33 36 20 33 0D | P | F |
|  | LLED LED Brightness | 1~100 | ~xx251 n | n=1-100 | 7E 30 30 32 35 31 20 31 0D to 7E 30 30 32 35 31 20 31 30 30 0D | P | F |

### GET Command

| Type | Category | CMD |  | Hex Code CMD | Response |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Success | Parameter | Failed |
| GET | Power | ~xx124 n | n=1 | 7E 30 30 31 32 34 20 31 0D | Ok0 | Power off | F |
|  |  |  |  |  | OK1 | Power on | F |
|  | Contrast | ~xx126 n | n=1 | 7E 30 30 31 32 36 20 31 0D | OK0-100 | 0-100 | F |
|  | Brightness | ~xx125 n | n=1 | 7E 30 30 31 32 35 20 31 0D | OK0-100 | 0-100 | F |
|  | Volume | ~xx120 n | n=1 | 7E 30 30 31 32 30 20 31 0D | OK0-10 | 0-10 | F |
|  | Mute | ~xx356 n | n=1 | 7E 30 30 33 35 36 20 31 0D | OK0 | Off | F |
|  |  |  |  |  | OK1 | On | F |
|  | Input Source | ~xx121 n | n=1 | 7E 30 30 31 32 31 20 31 0D | OK7 | HDMI1 | F |
|  |  |  |  |  | OK8 | HDMI2 | F |
|  |  |  |  |  | OK9 | HDMI3 | F |
|  |  |  |  |  | OK15 | DP | F |
|  |  |  |  |  | OK18 | 3G-SDI | F |
|  |  |  |  |  | OK21 | OPS | F |
|  |  |  |  |  | OK20 | Android | F |

| Type | Category | CMD |  | Hex Code CMD | Response |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Success | Parameter | Failed |
| GET | Aspect Ratio | ~xx127 n | n=1 | 7E 30 30 31 32 37 20 31 0D | OK1 | 4:3 | F |
|  |  |  |  |  | OK2 | 16:9 | F |
|  |  |  |  |  | OK7 | Auto | F |
|  | Display Mode | ~xx123 n | n=1 | 7E 30 30 31 32 33 20 31 0D | OK1 | Presentation | F |
|  |  |  |  |  | OK2 | Bright | F |
|  |  |  |  |  | OK3 | Cinema | F |
|  |  |  |  |  | OK21 | HDR | F |
|  |  |  |  |  | OK5 | User | F |
|  | Color Temp | ~xx128 n | n=1 | 7E 30 30 31 32 38 20 31 0D | OK3 | 3200K | F |
|  |  |  |  |  | OK0 | 6500K | F |
|  |  |  |  |  | OK1 | 9300K | F |
|  |  |  |  |  | OK2 | 12000K | F |
|  | WLAN IP address | ~xx451 n | n=2 | 7E 30 30 34 35 31 20 32 0D | Oknnn:nnn:nnn:nnn | IP address | F |
|  | LAN Mac address | ~xx555 n | n=1 | 7E 30 30 35 35 35 20 31 0D | Oknn:nn:nn:nn:nn:nn | Mac Address | F |
|  | LAN IP address | ~xx87 n | n=3 | 7E 30 30 38 37 20 33 0D | Oknnn:nnn:nnn:nnn | IP address | F |
|  | FW version | ~xx122 n | n=1 | 7E 30 30 31 32 32 20 31 0D | Oknnnnnnnn | FW version | F |
|  | MCU version | ~xx357 n | n=2 | 7E 30 30 33 35 37 20 32 0D | Oknnnnnnnn | Device type = DEVICE | F |
|  | Information string | ~xx150 n | n=1 | 7E 30 30 31 35 30 20 31 0D | OKabbbbbccddddee (Note*1) |  | F |
|  |  |  | n=2 | 7E 30 30 31 35 30 20 32 0D | Oknnn | Device native resolution | F |
|  |  |  | n=3 | 7E 30 30 31 35 30 20 33 0D | Oknnn (ex. OKHDMI1) | Input source | F |
|  |  |  | n=4 | 7E 30 30 31 35 30 20 34 0D | Oknnn (ex. OK1920x1080) | Source resolution | F |
|  |  |  | n=16 | 7E 30 30 31 35 30 20 31 36 0D | OK0 | Power mode (standby) = Eco. | F |
|  |  |  |  |  | OK1 | Power mode (standby) = Active | F |
|  |  |  | n=18 | 7E 30 30 31 35 30 20 31 38 0D | Oknnn (ex. OK48) | System temperature | F |
|  |  |  | n=19 | 7E 30 30 31 35 30 20 31 39 0D | Oknnn (ex. OK60Hz) | Source refresh rate | F |

| Type | Category | CMD |  | Hex Code CMD | Response |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Success | Parameter | Failed |
| GET | Regulatory Model name | ~xx151 n | n=3 | 7E 30 30 31 35 31 20 33 0d | Oknnn (ex. SLUGRK) | FHDQGSUS | F |

Note: (*1)

| Power | Running Time | Input Source | Firmware Version | Display Mode |
| --- | --- | --- | --- | --- |
| - a=0 Power Off | Usage hours = nnnnn | - cc=07 HDMI1 | # # # # | - ee=01 Presentation |
| - a=1 Power On |  | - cc=08 HDMI2 |  | - ee=02 Bright |
|  |  | - cc=09 HDMI3 |  | - ee=05 User |
|  |  | - cc=15 HDMI3 |  | - ee=03 Cinema |
|  |  | - cc=18 DP |  | - ee=21 HDR |
|  |  | - cc=21 3G-SDI |  |  |
|  |  | - cc=20 OPS |  |  |
|  |  | - cc=21 Android |  |  |

## LED Indicator

| Power LED Color & Behavior | Status |
| --- | --- |
| Solid Red | Standby mode |
| Solid Green | Power on |
| Flash Red (2 seconds) | Shutting down |
| Flash Green | Booting up |
| Flash Red (1 second) | Sender connection error |
| Flash Green and Red | SmartBorad Error |

# Optoma Global Offices

For service or support, please contact your local office

| USA |  | Japan |  |
| --- | --- | --- | --- |
| 47697 Westinghouse Drive, | 888-289-6786 | 東京都足立区綾瀬3-25-18 |  |
| Fremont, CA 94539, USA | 510-897-8601 | 株式会社オーエス | info@os-worldwide.com |
| www.optomausa.com | services@optoma.com | コンタクトセンター:0120-380-495 | www.os-worldwide.com |
| Canada |  | Taiwan |  |
| 47697 Westinghouse Drive, | 888-289-6786 | 12F., No.213, Sec. 3, Beixin Rd., | +886-2-8911-8600 |
| Fremont, CA 94539, USA | 510-897-8601 | Xindian Dist., New Taipei City 231, | +886-2-8911-6550 |
| www.optomausa.com | services@optoma.com | Taiwan, R.O.C. | services@optoma.com.tw |
|  |  | www.optoma.com.tw | asia.optoma.com |
| Latin America |  |  |  |
| 47697 Westinghouse Drive, | 888-289-6786 | Hong Kong |  |
| Fremont, CA 94539, USA | 510-897-8601 | Unit A, 27/F Dragon Centre, |  |
| www.optomausa.com | services@optoma.com | 79 Wing Hong Street, | +852-2396-8968 |
|  |  | Cheung Sha Wan, | +852-2370-1222 |
| Europe |  | Kowloon, Hong Kong | www.optoma.com.hk |
| Unit 1, Network 41, Bourne End Mills, |  |  |  |
| Hemel Hempstead, Herts, |  | China |  |
| HP1 2UJ, United Kingdom | +44 (0) 1923 691 800 | Room 2001, 20F, Building 4, | +86-21-62947376 |
| www.optoma.eu | +44 (0) 1923 691 888 | No. 1398 Kaixuan Road, | +86-21-62947375 |
| Service Tel : +44 (0)1923 691865 | service@tsc-europe.com | Changning District | servicecn@optoma.com.cn |
|  |  | Shanghai, 200052, China | www.optoma.com.cn |
| Benelux BV |  |  |  |
| Randstad 22-123 | +31 (0) 36 820 0252 | Korea |  |
| 1316 BW Almere | +31 (0) 36 548 9052 | WOOMI TECH.CO.,LTD. | +82+2+34430004 |
| The Netherlands |  | 4F, Minu Bldg.33-14, Kangnam-Ku, | +82+2+34430005 |
| www.optoma.nl |  | Seoul,135-815, KOREA |  |
|  |  | korea.optoma.com |  |
| France |  |  |  |
| Bâtiment E | +33 1 41 46 12 20 |  |  |
| 81-83 avenue Edouard Vaillant | +33 1 41 46 94 35 |  |  |
| 92100 Boulogne Billancourt, France | savoptoma@optoma.fr |  |  |
| Spain |  |  |  |
| C/ José Hierro,36 Of. 1C | +34 91 499 06 06 |  |  |
| 28522 Rivas VaciaMadrid, Spain | +34 91 670 08 32 |  |  |
| Deutschland |  |  |  |
| Wiesenstrasse 21 W | +49 (0) 211 506 6670 |  |  |
| D40549 Düsseldorf, | +49 (0) 211 506 66799 |  |  |
| Germany | info@optoma.de |  |  |
| Scandinavia |  |  |  |
| Lerpeveien 25 | +47 32 98 89 90 |  |  |
| 3040 Drammen | +47 32 98 89 99 |  |  |
| Norway | info@optoma.no |  |  |
| PO.BOX 9515 |  |  |  |
| 3038 Drammen |  |  |  |
| Norway |  |  |  |

www.optoma.com